IAEE conference in Istanbul, June 18-20 2008

Cost-effective solutions for CO2-neutral urban heating.

Anders Dyrelund,

Market Manager for Energy and Climate

Ramboll Denmark

Teknikerbyen 31

2830 Virum

Denmark

ad@ramboll.dk

1. Overview

The world is looking for cost-effective solutions for CO2 emission reduction. As Denmark will host the coming UN climate conference in Copenhagen in 2009 and as Denmark has achieved remarkable results in the past 25 years, it could be of interest to use this opportunity to study the special Danish experience. Denmark has managed to stabilise the gross energy consumption in the past 20 years while at the same time the GDP has grown 80%. Moreover, the share of renewable energy of the gross energy consumption has increased from 3 to 15%.

The main reason for the successful improvements in Denmark is a combination of technical, institutional and legal efficiency measures on both the supply side and the demand side in the heating sector.

The most important is not the technical measures. It is the legal and institutional approach combined with the Danish tradition for co-operation and entrepreneurship.

The most remarkable driver in this development is the district heating systems, which interconnects most consumers in each town or city with the most efficient heat sources. Since 1980 the market share of these systems in Denmark have increased from 30 to 60%, while at the same time the fossil fuel consumption for producing the heat has been reduced in average from 1.2 to 0.5 MWh fuel/MWh heat, delivered to the buildings. Ramboll estimates that the market share will grow to 70% in 2020 and that the fossil fuel consumption will be further reduced to 0.3 MWh/MWh.

In some towns the heating is even CO2 neutral due to use of heat from municipal waste and biomass. The Greater Copenhagen regional district heating system having a total heat production of 30.000 TJ has an efficiency of around 0.3 MWh fossil fuel/MWh heat. The CHP plants and the district heating in Copenhagen has in total a Primary Energy Factor of only 0.13 (according to Euroheat methodology).

2. Methods

Some important technical measures are:

- • Each building has installed a based water central heating system and a system for distribution of hot tap water. The temperatures are typically 80 dgr.C supply and 40 dgr.C return.

- • In almost all urban districts with a certain heat density a water-based district heating system has been established, which interconnects all buildings with the most efficient heat production plants. In districts with less heat density individual solutions are applied.

- • The connection rate to the district heating networks is typically between 80% and 99%. In the largest municipality, Copenhagen, 98% of all buildings in the municipality are connected.

- • The heat production is optimised hour by hour giving first priority to surplus heat from CHP plants and renewable energy. Less than 5% of the heat is produced by fossil fuel boilers. The district heating company buys the cheapest heat and supplements with own production if necessary on behalf of all the consumers.

- • New power generation capacity has since 1976 been designed as CHP plants and situated close to the cities with the option to supply heat in cogeneration with electricity.

- • All CHP plants have installed large heat accumulators (typically with a capacity equal to 8 hours of max load) to optimise the production of heat and electricity.

Some important institutional measures are:

- • All apartment buildings with privatised apartments have organised a condominium, which is a co-operative of apartment owners.

- • Most district heating companies are co-operatives owned by the consumers (building owners), which are supplied. The rest are, with a few exceptions, owned by municipalities, but the municipalities operate the companies on behalf of the consumers.

- • District heating transmission systems, which supplies more than one municipality are normally owned by a co-operative of the municipalities, which are supplied.

We can say that the consumers have organised themselves in co-operatives on several levels (building, distribution, production and transmission), all with the aim to serve the interests of the consumers. The companies are “non-profit” companies. This turns out to be the most efficient ownership form for such natural monopoly networks, as the financial costs are the lowest possible and as the consumers control the companies.

Some important legal measures are:

- • All privatised apartments are obliged to be member of the condominium of the building and follow all democratic decisions.

- • All municipalities shall include heat planning as part of the urban planning and e.g. approve the zoning of district heating and natural gas networks for heating. A result is that district heating is only established, if it is the most cost-effective solution compared to individual solutions.

- • The municipality can decide that buildings shall connect and remain connected to the district heating network, which is regarded as a natural part of the urban infrastructure, like roads, sewage, water etc.

- • The heat price can only include the necessary costs, and all profit shall be transferred back to the consumers.

- • In case a district heating network is sold, the consumers and the municipality shall have the first right to buy the network.

- • The government tries to promote the best solutions by combining regulatory measures with tax incentives and certain subsidies, e.g. to electricity produced by renewable energy.

3. Conclusion

The more than 100 years tradition of forming private co-operatives and the long-term commitment of the parliament and city councils have been important preconditions for the successful Danish CO2 emission reductions in the heating sector and for further reductions.