THE COSTS OF POLICIES SUPPORTING A QUICK TRANSITION TO FUEL CELL VEHICLES

Dogan Keles, Institute for Industrial Production (IIP), Universität Karlsruhe (TH), +49-721-6084566, dogan.keles@iip.uka.de

Martin Wietschel, 2Fraunhofer Institute for Systems and Innovation Research (ISI), +49-721-6809254, matin.wietschel@isi.fhg.de

Dominik Möst, Institute for Industrial Production (IIP), Universität Karlsruhe (TH), +49-721-6084689, dominik.moest@iip.uka.de

Otto Rentz, Institute for Industrial Production (IIP), Universität Karlsruhe (TH), +49-721-6084460, otto.rentz@iip.uka.de

## Overview

Hydrogen fuel is one of the most important research areas aiming to shift to “clean fuels” in the transport sector. There are several economic studies and models describing the introduction of hydrogen driven vehicles, but most of them are focused on one part of the car market. Most studies analyse separately the impacts of fuel cell vehicles (FCV) on the automotive industry or the demand for FCVs, while other studies look at the need for hydrogen infrastructure, but all of these analyses do not have a view on the complete car market and the transportation system. As a part of the EU-project MATISSE (www.matisse-project.net), this analysis discusses the market penetration of fuel cell vehicles (FCV) in Germany on perspectives of all main car market stakeholders (consumers, automotive manufacturers, fuel station owners and policy makers).

## Methods

The analysis is based on the System Dynamics (SD) methodology. According to the system dynamics society, SD is a methodology for studying and managing complex feedback systems, such as one finds in business, economics and other social systems [2]. Based on the SD methodology a consumer decision model was developed. Within the consumer decision model the actors of the transport system and of the car market are reproduced. Their relations and impacts on each other are also modelled, abstracting them in mathematical equations. The four modules of the model are focusing on 1) the infrastructure build up, 2) the attractiveness of the FCVs, 3) the FCV stock development and at last on 4) the subsidy or tax policies. These four modules include also main feedback loops (i.e. causal loop) of a hydrogen transportation system (FCV stock influences () number of filling stations  FCV demand  FCV production  FCV sales  FCV stock). The main advantage of the SD methodology is the easy and clear reproduction of dynamic systems using graphical illustrations as well as mathematical equations for the relations between system elements.

## Results

The policy-mix and the assumptions in the lead scenario are defined so that the market penetration of FCVs succeeds within an acceptable time-period. In fact the FCV stock increases from the starting year 2013 until 2040 to an amount of more than 17 millions vehicles in Germany, i.e. more than one third of all passenger cars.

Fig. 1: Simulation results of the Lead Scenario (high growth scenario due to favourable policies) – 
           		 FCV and filling station development

The development of the FCV stock conforms at first to that of exponential growth, as it is usual for new technologies (Fig. 1). Later, after 2040, it is assumed that the growth of the FCV stock is a digressive one approximating to the upper limit “total car stock”.

In order to the learning effects in the production caused by a higher production rate the costs of FCVs sink vastly to the level of diesel cars, so that the state has to subsidy the FCVs only for a short time. Little amount of subsidies are necessary for the build-up of hydrogen filling stations in urban areas as well as on the highways. The highest support is needed in the area of vehicle cost reduction, especially through vehicle subsidies and VAT abandon at the introduction phase. Besides, the State has to accept high “costs” caused by abstaining from hydrogen fuel taxes to make FCVs more attractive. However, the total amount of subsidies and tax deficit, i.e. the total costs for the State, are limited up to approx. five billion Euros spread over eight years in the lead scenario.

Fig. 2: Simulation results of the Lead Scenario (high growth scenario due to advantageous policies) –  subsidies and tax deficit

If the state is not willing to give vehicle subsidies or to support the infrastructure build-up, the penetration of FCVs precedes very slowly. In this case the vehicle cost reduction due to learning effects will be at a low level, so the State has to abandon vehicle and hydrogen taxes for a longer time period in order to make FCVs somehow attractive. This means that the overall costs for the State are in these scenarios even higher than in the lead scenario. Besides, in scenarios without subsidies for FCVs and hydrogen filling stations the switch to an environmentally clean transportation system will last a long time.

## Conclusions

The transition to hydrogen as an energy fuel in the transport sector is a crucial innovation and relevant support is needed in different areas for a limited time period. Without any support, hydrogen will enter the market very slowly. But after a short period of support FCVs can lead to less total costs than conventional vehicles and the penetration of FCVs and of hydrogen as fuel would expedite. Further it is important to reach very fast a certain level of market penetration of hydrogen vehicles and infrastructure build up, because the overall financial support is much higher in scenarios of a slow market penetration than in scenarios with a fast market penetration. If a quick transition of FCVs is not possible, it is very difficult that hydrogen as an alternative fuel succeeds.

## References

[1] Matisse - Methods and Tools for Integrated Sustainability Assessment, Integrated Project under the 6th Framework Programme of EU, (www.matisse-project.net); 2007
[2] Forrester, Jay W., Industrial Dynamics. M.I.T. Press, Cambridge, Mass. 1977
[3] Schwoon, Malte, Simulating the adoption of Fuel Cell Vehicles, Working Paper FNU-59, Centre for Marine and Climate Research, Hamburg University, 2006
[11] HyWays, HyWays-hydrogen energy in Europe, Integrated Project under the 6th Framework Programme of the European Commission, 2007-2007, (www.hyways.de); 2007