An Algorithm for Voltage Uprating in a Subtransmission Network

| Asghar Akbari Foroud Semnan University Semnan/Iran aakbari@semnan.ac.ir +982313334201 | Hossein Seifi Tarbiat Modares University Tehran/Iran seifi_ho@modares.ac.ir +982188004556 | Khodarahm Asiaei Fars Regional Electric Company (FREC) Shiraz/Iran asiaei.kh@gmail.com +987112359052 |
| --- | --- | --- |

1) OVERVIEW

Due to electric demand growth, power system transfer capability needs to be increased. Impacts of land use, transmission line visual impacts, suspected health effects from EMF and environmental impacts, limit the constructing of new lines [1,2,3]. On the other hand, investing on transmission line construction projects lacks economical attractions [4]. Therefore, some other ways of increasing the power transfer capability such as uprating of existing lines; by either increasing the size or the number of conductors and using high temperature-rated conductors; dynamic thermal rate monitoring and voltage uprating should be, initially, investigated and can be practical solutions [3-6].

In this paper, an algorithm is presented in which voltage uprating is employed and compared, technically and economically, with constructing new lines in a subtransmission network.

2) METHOD

The following algorithm is proposed for justification of voltage uprating of a subtransmission network. Initially, the cost of network expansion in existing voltage is evaluated (section 2-1). Thereafter, the same cost is calculated upon using voltage uprating (section 2-2). These two are compared for final decision.

2-1) Network Expansion in Existing Voltage

Network expansion costs in existing voltage level consist of the cost of adequacy uprating of the existing network, the cost of substation expansion, the cost of transmission expansion and the cost of power losses, each of which with its own mathematical formulas and descriptions (developed & presented in full paper).

2-2) Cost of Voltage Uprating and Development of Subtransmission Network

The cost of voltage uprating and development of a subtransmission network is the sum of voltage uprating cost of existing network (from 66 to 132 kV), network expansion cost to meet the voltage drop constraint, new substations and lines costs and power losses cost, each of which with its own mathematical formulas and descriptions (developed & presented in full paper).

3) NUMERICAL RESULTS

The proposed algorithm is tested on a regional subtransmission network in Iran.

3-1) Development of the Network in 66 kV

Using the mathematical formulas developed & presented in the full paper, network expansion cost in 66 kV is calculated to be: 80000  106 R (where R represents Iranian monetary unit).

3-2) The Cost of Voltage Uprating and Development of The Network in 132 kV

Using the mathematical formulas developed & presented in the full paper, network expansion cost in 66 kV is calculated to be: 78994  106 R.

By Comparing of the costs, it is evident that voltage uprating of this network is cost benefit.

4) CONCLUSION

Voltage uprating is one of the power network transfer capability increasing methods. It is an expensive option, so that in short time, it isn't normally justified. On the other hand, determining network expansion cost in long term period is impossible. In this paper an algorithm was presented which compares the costs of two scenarios; constructing new lines, in existing voltage (ex. 66 kV)  and voltage uprating of the network (for ex. from 66 kV to 132 kV). This algorithm was tested on a 18 bus subtransmission network.

5) REFERENCES

[1] C. E. Williamson, "Transmission Line Developments up to 150 kV in Eastern Australia", International Conference on Overhead Line Design and Construction: Theory and Practice, UK, 1989, pp. 173-177.

[2] J. G. Hanson, "Upgrading Transmission Lines", Transmission and Distribution Conference, 1991., Proceedings of the 1991 IEEE Power Engineering Society, Dallas, TX, USA, pp.824-827.

[3] I. Zamora, A. J. Mazon, R. Criado, C. Alonso and J. R. Saenz, "Uprating Using High-Temperature Electrical Conductors", 16th International Conference and Exhibition on Electricity Distribution (IEE Conf. Publ. No. 482), 2001. Amsterdam, Volume: 1.

[4] J. R. Daconti and D. C. Lawry, "Increasing Power Transfer Capability of Existing Transmission Lines", Transmission and Distribution Conference and Exposition, 2003 IEEE PES, Schenectady, NY, USA, pp. 1004-1009.

[5] I. T. Domingues, J. C. R. Lopes, R. Roqelane, S. Cabral and S. Ueda, "Uprating of  Transmission Lines-Reconductoring ACSR by Conductors Thermo-Resistant TACSR and TACIR (Invariable)", Transmission and Distribution Conference and Exposition: Latin America, 2004 IEEE/PES, AES, Eletropaulo, Brazil, pp. 723-726.

[6] I. Zamora, A. J. Mazon, P. Eguia, R. Criado, C. Alsono, J. Iglesias and J. R. Saenz, "High-Temperature Conductors: A Solution in the Uprating of Overhead Transmission Lines", Power Tech Proceedings, 2001 IEEE Porto, Porto, Portugal, Volume: 4.