EVALUATING ECOSYSTEM AND WIND-FOLLOWING SERVICES FOR HYDROELECTRIC DAMS IN PJM

Alisha Fernandez, The Pennsylvania State University, (206) 491-0392, arf176@psu.edu

Seth Blumsack, The Pennsylvania State University, (814) 863-7597, blumsack@psu.edu

Patrick Reed, The Pennsylvania State University, (814) 863-2940, preed@engr.psu.edu

## Overview

Hydropower can provide inexpensive, flexible fill-in power to compensate for intermittent renewable generation. Policies for hydropower dams maintain multiple services beyond electric generation, including environmental protection, flood control and recreation. We model the decision of a hydroelectric generator to shift some of its power production capacity away from the day-ahead energy market into a “wind-following” service that smoothes the intermittent production of wind turbines.  Offering such a service imposes both private and social opportunity costs.  Since fluctuations in wind energy output are not perfectly correlated with day-ahead energy prices, a wind-following service will necessarily affect generator revenues. Seasonal wind patterns produce conflicts with the goal of managing rivers for “ecosystem services” – the maintenance or enhancement of downstream ecosystems.  We illustrate our decision model using the Kerr Dam in PJM’s territory in North Carolina.  We simulate the operation of Kerr Dam over a three-year period that features hydrologic variability from normal water years to extreme drought conditions. We use an optimization framework to estimate reservation prices for Kerr Dam offering wind-following services in the PJM market.  Wind-following may be profitable for Kerr Dam at low capacity levels during some time periods if ecosystems services are neglected and if side payments, or reserves-type payments, are provided. Wind-following with ecosystem services yields revenue losses that typically cannot be recovered with reserves market payments.  Water release patterns are inconsistent with ecosystem-services goals when Kerr Dam dedicates significant capacity to wind-following, particularly in drought years.

## Methods

We use an optimization framework and scenario analysis to model operational decisions for a small-scale hydroelectric dam in a water-constrained geography. We develop policy scenarios that highlight distinct services the dam provides and illustrate the social trade-offs involved in managing river systems, including revenue impacts to power producers and ecosystem services.

## Results

First, market prices for ancillary services in PJM would not need to increase to incentivize Kerr Dam to shift capacity from the day-ahead market to the ancillary services market to offer wind-following services.

Second, reserve payments, however, steadily decrease with increased hydrological variability.

Third, a sensitivity analysis of the wind-following capacity amount sold at current ancillary (reserves) services market prices suggests that wind-following may be profitable for Dominion at low capacity levels, yet wind-following coupled with a policy goal of regulating flows for downstream ecosystem maintenance yields revenue loss that typically cannot be recovered with current reserves market payments.

## Conclusions

A significant policy tradeoff exists between the utilization of hydroelectric dams to provide ecosystem services and wind-following services. Water release patterns from Kerr Dam are inconsistent with the ecosystem-services goals when Dominion dedicates a significant portion of Kerr’s capacity to wind-following, particularly in drought years. More intense hydrological variability may require a stronger financial incentive for hydroelectric dam operators to further constrain their water supplies with wind-following and ecosystem services in the future based on current reserve market prices.

## References

Apt J., Keith, D.W., and Morgan, M.G. Promoting Low-Carbon Electricity Production [Journal]. - [s.l.] : Issues in Science & Technology , 2007. - Vols. 24, no. 3: 37-44.

Apt. J. The spectrum of power from wind turbines [Journal]. - [s.l.] : Journal of Power Sources, 2007. - Vols. 169:2, pp. 369-374.

De Ladurantaye D., Gendreau, M., and Potvin, J. Optimizing profits from hydroelectricity production [Journal]. - [s.l.] : Computers and Operations Research, 2007. - Vol. 36.

Harou J., Pulido-Velazquez, M., Rosenberg, D., Medellin-Azuara, J., Lund, J. and Howitt, R. Hydro-economic models: Concepts, design, application, and future prospects [Journal]. - [s.l.] : Journal of Hydrology, 2009. - Vols. 375, pp. 627-643.

Horsley A. and Wrobel, J. Efficiency Rents of Storage Plants in Peak-Load Pricing, II: Hydroelectricity [Journal]. - [s.l.] : London School of Economics and Political Science, 1999. - Vol. Discussion Paper No. TE/99/372.

Katzenstein W. and Apt, J. Air Emissions Due to Wind and Solar Power [Journal]. - [s.l.] : Environmental Science and Technology, 2009. - Vols. 43 (2): 253-258.

Kern J.D. Influence of De-regulated Electricity Markets on Hydropower Generation and Downstream Flow Regime [Thesis]. - 2010.

Li Y., Blumsack, S., and Xu, J. Implications of Renewable Electricity Generation Siting on Air Emissions in the Western U.S. [Journal]. - [s.l.] : Energy Policy, (in press) 2011.

Ozelkan E., Galambosi, A., Fernandez-Gaucherand, E. and Duckstein, O. Linear quadratic dyanmic programming for water reservoir management [Journal]. - [s.l.] : Applied Mathematical Modeling, 1997. - Vols. 21: pp. 591-598.

Perekhodtsev D. Two Essays on Problems in Electricity Deregulation, Ph.D. Dissertation. - [s.l.] : Carnegie-Mellon University, 2003.

Pindyck R. An Application of Linear Quadratic Tracking Problem to Economic Stabilization Policy [Journal]. - [s.l.] : IEEE, 1972. - Vols. 17, No. 3: pp. 287-300.

Richer A., Baumgartner, J., Braun, D. A Method for Assessing Hydrologic Alteration within Ecosystems [Journal]. - [s.l.] : Conservation Biology, 1996. - Vols. 10, No.4: pp. 1163-1174.

Seager R., Tzanova, A. and Nakamura, J. Drought in the Southeastern United States: Causes, Variability over the last Millennium, and the Potential for Future Hydroclimatic Change [Journal]. - [s.l.] : Journal of Climate, 2009. - Vols. 22, pp. 5021:5045.

Simonovic S. Reservoir Systems Analysis: Closing Gap Between Theory and Practice [Journal]. - [s.l.] : Journal of Water Resources Planning and Management, 1990. - Vols. 118, No. 3.

Sims G. Hydroelectric Energy [Journal]. - [s.l.] : Energy Policy, 2003. - 8 : Vols. 18: pp. 776-786.

Sioshansi R., Denholm, P., Jenkin, T., and Weiss, J. Estimating the Value of Electricity Storage in PJM: Arbitrage and Some Welfare Effects [Journal]. - [s.l.] : Energy Economics, 2009. - Vols. 31, no. 2: pp. 269-277.

Whisnant R.B., Characklis, G.W., Doyle, M.W., Flatt, V.B., and Kern, J.D.. Operating policies and administrative discretion at the John. H. Kerr project [Report]. - 2009.

Zhang J.L. and Ponnambalam, K. Hydro energy management optimization in a deregulated electricity market [Journal]. - [s.l.] : Optimization and Engineering, 2002. - Vols. 7, Number 1, 47-71.